EE Broth Mossel

E E Broth, Mossel is used for selective enrichment of Enterobacteriaceae in bacteriological examination of foods.

| REF: V.1/EE01.100 100 Gram REF: V.1/EE01.500 500 Gram | REF: V.1/EE01.250 250 Gram |
| --- | --- |

# CLINICAL SIGNIFICANCE

The family Enterobacteriaceae consists of Salmonella, Shigella and other enteric pathogens. These organisms find entry into the food system through faecally contaminated water. Majority of these organisms may be eliminated under the stringent food processing parameters. But some of these organisms may become sublethally injured during the changes in pH, exposure to steam or heat and other unfavourable conditions (1). Therefore the important aspect of food monitoring depends upon the identification and enumeration of these injured cells, after resuscitation. EE Broth, Mossel, formulated by Mossel et al (2) is recommended as an enrichment medium for Enterobacteriaceae in the biological examination of foods (2) and animal feed stuffs (3).

METHOD PRINCIPLE

# Peptic digest of animal tissue and dextrose provide the essential nutrients required for the growth of most of the members of Enterobacteriaceae. Brilliant green and ox bile, purified inhibit growth of gram-positive bacteria. Lactose-negative, anaerogenic lactose-positive or late lactose-fermenting Enterobacteriaceae are often missed by the standard coli-aerogenes test. To overcome this problem, lactose is replaced by dextrose in these media. Phosphates form the buffering system of the medium. The cells damaged while drying or low pH are resuscitated in well-aerated Tryptone Soya Broth for 2 hours at 25°C prior to enrichment in EE Broth. The resuscitation procedure is recommended for dried foods (5), animal feeds (6) and semi-preserved foods (7). EE Broth is an enrichment broth and should be used in conjunction with Violet Red Bile Glucose Agar. Subcultures must be made onto lactose differential media such as MacConkey Agar, Deoxycholate Citrate Agar or Brilliant Green Agar for the detection of lactose negative or delayed lactose fermenters. This is used to inoculate MPN tubes prepared using EE Broth. Inoculate a loopful from these tubes onto Violet Red Bile Glucose Agar after an initial incubation at 35-37°C for 24 hours. Typical pink colonies from M581 are subcultured for biochemical confirmation by oxidase and fermentation reactions (4). Decimal dilutions of the food homogenate are used if the expected counts are high or else initial suspension is used.EE Broth, Mossel.

# MEDIA COMPOSITION

| Item | Formula per liter of medium |
| --- | --- |
| - Peptic digest of animal tissue - Dextrose - Disodium phosphate - Monopotassium phosphate - Ox bile, purified - Brilliant green | 10 5 6.425 2 20 0.0135 |

PRECAUTIONS AND WARNINGS

Media to be handled by entitled and professionally educated person. Do not ingest or inhale.

Good Laboratories practices using appropriate precautions should be followed in:

- Wearing personnel protective equipment (overall, gloves, glasses,).

- Do not pipette by mouth.

- In case of contact with eyes or skin; rinse immediately with plenty of soap and water. In case of severe injuries; seek medical advice immediately.

- Respect country requirement for waste disposal.

S56: dispose of this material and its container at hazardous or special waste collection point.

S57: use appropriate container to avoid environmental Contamination.

S61: avoid release in environment.

For further information, refer to the EE Broth material safety data sheet.

# STORAGE AND STABILITY

Lab.Vie.EE Broth dehydrated media are stable until expiration date stated on label when properly stored 10-30°C. The prepared medium should be stored at 2-8°C. Use before expiry date on the label. On opening, product should be properly stored dry, after tightly capping the bottle in order to avoid lump development due to the hygroscopic nature of the product. Improper storage of the product may lead to lump formation. Store in a dry ventilated area protected from extremes of temperature and sources of ignition. Seal the container tightly after use. Product performance is best if used within stated expiry period.

Final pH 7.2 ± 0.2 at 25°C

# MEDIA PREPARATION

- Suspend 43.46 grams in 1000 ml distilled water.

- Adjust pH to 7.2 ± 0.2 at 25°C.

- Dispense in tubes or flasks as desired.

- Stopper with cotton plugs or loose fitting caps.

- Heat in free flowing steam or boiling water for 30 minutes.

- Avoid overheating of the medium. DO NOT AUTOCLAVE.

Deterioration

The color of Lab.Vie.EE Broth is Light yellow to greenish yellow homogeneous free flowing powder. If there are any physical changes, discard the medium.

The hydrated medium is Emerald green coloured, clear solution without any precipitate, media should not be used if there are any signs of deterioration (shrinking, cracking, or discoloration), and contaminations.

# SPECIMEN COLLECTION AND PRESERVATION

# Food and food-stuff

# EQUIPMENT REQUIRED NOT PROVIDED

- Sterile cups

- Sterile tubes

- Sterile loops

- Incubator

# PERFORMANCE CHARACTERISTICS

Performance of the medium is expected when used as per the direction on the label within the expiry period when stored at recommended temperature

| SYMBOLS IN PRODUCT LABELLING |  |  |
| --- | --- | --- |
| IVD | For in-vitro diagnostic use | Number of <n> test in the pack |
| LOT | Batch Code/Lot number | Caution |
| REF | Catalogue Number | Do not use if package is damaged |
|  | Temperature Limitation | Consult Instruction for use |
|  | Expiration Date |  |
|  | Manufactured by |  |

| Test Organisms | Growth | Acid |
| --- | --- | --- |
| Escherichia coli ATCC 8739 | Luxuriant | positive reaction, yellow colour |
| Pseudomonas aeruginosa ATCC 9027 | Luxuriant | - |
| Staphylococcus aureus ATCC 6538 | Inhibited | - |
| Escherichia coli ATCC 25922 | Luxuriant | positive reaction, yellow colour |
| Escherichia coli NCTC 9002 | Luxuriant | positive reaction, yellow colour |
| Pseudomonas aeruginosa ATCC 27853 | Luxuriant |  |
| Enterobacter aerogenes ATCC 13048 | Luxuriant | positive reaction, yellow colour |
| Proteus mirabilis ATCC 25933 | Luxuriant | positive reaction, yellow colour |
| Salmonella Enteritidis ATCC 13076 | Luxuriant | variable reaction |
| Shigella boydii ATCC 12030 | Luxuriant | negative reaction |
| Staphylococcus aureus ATCC 25923 | Inhibited | - |

# Cultural response was observed after an incubation at 35-37°C for 24-48 hours.

# QUALITY CONTROL

To ensure adequate quality control, it is recommended that positive and negative control included in each run. If control values are found outside the defined range, check the system performance. If control still out of range please contact Lab.Vie. technical support.

# REFERENCES

1. Mossel D. A. A., and Harrewijn G. A., 1972, Alimenta II, 29-30

2. Mossel D. A. A., Vissar M. and Cornellisen A. M. R., 1963, J. Appl.Bacteriol., 26(3):444.

3.Van Schothurst M. et al, 1966, Vet Med., 13(3):273.

4. International Organization for Standardization (ISO), 1993, Draft ISO/DIS 7402.

5. Mossel D. A. A. and Ratto M. A., 1970, Appl. Microbiol., 20:273.

6. Mossel D. A. A., Shennan J. L. and Clare V., 1973, J. Sci. Fd. Agric., 24: 499.

Ismailia – Free zone, Ismailia – Egypt                                                         IFU-S-02, Rev. 03 - December 2019

Post code-4151

E-mail   : admin@labvielab.com

Website: www.labvielab.com

7. Mossel D. A. A. and Ratto M. A., 1973, J. Food Technol., 8: 97.